Note: This template provides the minimum information elements for an SDS required by Schedule 1 of the Hazardous Products Regulations (HPR) (WHMIS 2015)
 Safety Data Sheet

Bartlett Mouse Bait
	SECTION 1. IDENTIFICATION


	Product Identifier:
	Bartlett Mouse Bait

	Registration Number:
	PCP# 8024

	Product Used:
	Rodent Bait

	Synonym:
	Not Available

	Initial Supplier:
	N.M. Bartlett Inc.

4509 Bartlett Rd

Beamsville, ON Canada
L0R 1B1
(905)563-8261

	Emergency Telephone Number:
	CANUTEC (613) 996-6666

	
	


	SECTION 2. HAZARD IDENTIFICATION


	Classification Label Element:
[image: image1.png]


 [image: image2.jpg]


 [image: image3.png]



	

	
	

	SECTION 3. COMPOSITION/INFORMATION ON INGREDIENTS


	Chemical Name

	Common Name

	CAS Number

	Concentration

	40mg/kg


	Trizinc diphosphae

	Zinc Phosphide

	1314-84-7

	2%

	Oral-Rat (M)


	Cracked Grain Corn

	Corn

	Not applicable 

	Trade secret

	

	
	
	


	SECTION 4. FIRST AID MEASURES


	Inhalation: 
Not applicable
	

	Skin Contact: 
Take off contaminated clothing. Wash skin with soap and water.
	

	Eye Contact: 
Flush eyes for 15-20 minutes. Remove contact lenses, if present, after 5 minutes, then continue rinsing eye.
	

	Ingestion: 
Call doctor for immediate treatment advice. Sip water slowly if you are able to swallow. Do not induce vomiting, unless told otherwise. 
	

	
	

	
	

	SECTION 5. FIRE - FIGHTING MEASURES


	Flammability: Very slowly gives off flammable. Poisonous phosphine gas when exposed to water, rapidly gives

off phosphine gas when exposed to acid.

	Flammable Limits: Not established
	

	Flash Point: Not established
	

	Auto-Ignition Temperature: Not Established
	

	Suitable and Unsuitable Extinguished Media: Use dry chemical, foam or CO2 extinguishing media. If water

is to be used to fight fire, bank and collect run-off.

Specific Hazards: During a fire, irritating and possible toxic gases may be generated by thermal decomposition of fire.
Special Protective Equipment and Precautions for Fire Fighters: Put on full uniform of protective clothing and self-contained breathing device. Vacate extra personnel from the area to prevent human expose to fire, smoke, fumes or product of combustion.
	


Reactivity and under what Conditions: Reacts very slowly with water or moisture. Reacts violently with dilute or concentrated acids.
	SECTION 6. ACCIDENTAL RELEASE MEASURES


	Material Release or Spill: 
Avoid skin contact and breathing dust. Sweep up spilled material and empty into a dry container.

	

	
	

	SECTION 7. HANDLING AND STORAGE


Handling: 
Wear protective clothing and approved masks should be worn. Eating, drinking and smoking should be prohibited
when handling and also before washing after handling.

Storage: 
Keep out of reach of children, pets and from foodstuffs, animal feed and their containers, Keep dry and away
from all acids.

Applicators and other Handlers:

Special clothing, gloves and masks should be worn when being in contact.
	SECTION 8. EXPOSURE CONTROLS/PERSONAL PROTECTION


Individual Protection:

[image: image4.jpg]ACAUTION

Safety

glasses
must be worn
at all times






 INCLUDEPICTURE "https://store.thinksai.com/images/clipart/Safety%20First/Caution/Horizontal/cautH319%20-%20wear%20mask%20respiratory%20irritants%20in%20this%20.jpg" \* MERGEFORMATINET [image: image5.jpg]A CAUTION

Wear mask

Respiratory
irritants
in this area




[image: image6.jpg]



	
	

	
	

	Person Protection:

Eyes: Eye guard is not required for normal handling. Where eye contact is likely, shield with snug-fitting chemical 
goggles. 
Respiratory:

Respiratory protection is not required for normal usage. In the event of an unusual dust contact, use engineering
controls or a NIOSH approved particulate respirator to keep contact below the occupational exposure limit.
Skin:

Gloves are not required for normal usage. Where heavy contact is likely, shield in a chemical resistant glove.

Ingestion: 

Avoid eating, drinking, tobacco usage and cosmetic application in areas where there is a potential for exposure to

the material. Rinse thoroughly with soap and water after contact.

	

	
	


	SECTION 9. PHYSICAL AND CHEMICAL PROPERTIES


	Appearance: Pungent
	

	Odour: Garlic Odor
	

	Odour Threshold:  1.5-3.0 PPM (phosphine gas)
	

	Boiling Point: 1100 deg. C (A.I.)
	

	Evaporation Rate: Not established
	

	Upper Flammability (solid, gas):  Not established
Lower Flammable Limit: 2% (Phosphine Gas)
	

	Vapour Pressure: Not established
	

	Vapour Density:  Not established
	

	Pour Point: -Not established
	

	Solubility: Negligible. Product slowly decomposes in water
	

	Melting Point: >420°F (Zinc Phosphide Technical)
Freezing Point: 420 deg. C (A.I.)
	

	Relative Density: 41.00-44.00 lbs/ft³
Penetration: Not established
Oil/Water Dist. Coefficient: Not established
Iconicity (in water): Not established
Dispersion Properties: Not established
Physical State: Solid

pH: Not established

Specific Gravity: 4.7 g/ccm (A.I.)

	

	
	

	SECTION 10. STABILITY AND REACTIVITY

	Reactivity: Reactive with acids and water.
Corrosively: Note established
	

	Chemical Stability: Steady under common use and stowing conditions.
	

	Hazardous Polymerization:  Will not occur
	

	Conditions to Avoid: Avoid heat and incompatible materials. 

Incompatible Substances/: Acids and including dilutes. Water, may react with oxidizers.
	

	Decomposition Products: Might react with acids or water to release spontaneously flammable phosphine gas.
	


	SECTION 11. TOXICOLOGICAL INFORMATION


Likely Routes of Entry:
 x   Inhalation      Skin contact      Eye contact    x    Ingestion
	Acute Oral: LD 50 (Rat) – 910 mg/kg body weight
Acute Dermal:  LD 50 (Rabbit) - >2000 mg/kg body weight
Acute Inhalation: LC 50 (Rat) - <1 mg/kg body weight  
Skin Sensitization: Not established

	Skin Irritation: Moist skin and mucous membranes
	

	Eye Irritation: May cause irritation to eyes.
Carcinogenicity Potential: Not established
Mutagenicity: Not established 

Synergistic Products: Phosphine gas generated on contact with mucous membranes

Chronic Contact to Product: Nausea, abdominal pain, diarrhea and vomiting
Acute Contact to Product: After ingestion – nausea, abdominal pain, chest pain, chest tightness, excitement

agitation, chills, fatigue, shock, early dyspnoea, thirst, oliguria, convulsions, tremors, change in skin color.
	

	
	

	SECTION 12. ECOLOGICAL INFORMATION
Do not discharge effluent containing this product directly to water. Do not contaminate water when disposing

of equipment washout water.


	

	
	

	SECTION 13. DISPOSAL CONSIDERATIONS 


Do not reuse empty container. Dispose of unused or spoiled bait in accordance with local requirements. Follow

provincial instructions for any required cleaning of the container prior to its disposal. Make the empty container
unsuitable for further use. Dispose of the container in accordance with provincial requirements. For more information on the disposal of unused, unwanted product contact the manufacturer of the provincial regulatory

agency. Contact the manufacturer and the provincial regulatory agency in case of a spill, and for clean-up of

spills.
	SECTION 14. TRANSPORT INFORMATION 


DOT/TDG Shipping Description: Not Regulated
IMDG Shipping Description: Not Regulated

	
	


	SECTION 15. REGULATORY INFORMATION


Caution:

Harmful if swallowed or absorbed through skin. Encourage children and pets to stay away. Try not to get into

your eyes, on skin or on clothing.
	NFPA & HMIS Hazard Ratings:
NFPA

HMIS

1 – Health
1*- Health
0 - Minimal
1 – Flammability
1 - Flammability
1 - Slight
2 – Instability
2 - Reactivity
2 - Moderate
3 - Serious
4 - Extreme
*- Chronic


	


Reportable Quantity (RQ) under U.S. CERCLA: Not Listed

SARA, Title III, Section 313:  Zinc Phosphide (CAS 1314-84-7)
Section 311/312 Hazard Classes: Acute Health Hazard
RCRA Waste Code: U249 (for concentrations 10% or less)
CA Proposition 65:  Not Listed  
CERLA/SARA 302 Reportable Quantity (RQ): 100lbs of 100% Zinc Phosphide Technical
	SECTION 16. OTHER INFORMATION


For additional information, contact your technical Sales Representative. For additional

Health & Safety information, call N.M. Bartlett Inc. at 905-563-8261.

The information contained herein is based on the data available to us and is believed to be correct. However, N.M. Bartlett Inc. makes no warranty, expressed or implied, regarding the accuracy of this data or the results to be obtained from the use thereof.

N.M. Bartlett Inc. assumes no responsibility for injury from the use of the product described herein.

	Date of Latest Revision: April 4, 2018
	


